Weekly Work Report 6/14/2024

This Week:

- Summary Table for Previous Results

- Working on the EEG Dataset

  - Literature Review:

    - Chaddad, A., Wu, Y., Kateb, R., & Bouridane, A. (2023). Electroencephalography Signal Processing: A Comprehensive Review and Analysis of Methods and Techniques. Sensors, 23(14), 6434

    - H. Cheng, M. Wang, C. Ma and C. Yu, "A Review of Brain Information Processing for Robot Control," 2022 7th International Conference on Image, Vision and Computing (ICIVC), Xi’an, China, 2022, pp. 866-871, doi: 10.1109/ICIVC55077.2022.9886157.

- The characteristics of EEG signals

- Signal Non-linearity:

  - EEG signals exhibit non-linearity, activeness, and time-varying causality. They represent a complex, dynamic system with non-linear behavior.

  - This complexity arises from the interactions of neural populations, synaptic activity, and various physiological processes.

- Weakness of Signal:

  - Normal EEG signals are indeed weak. The spontaneous EEG potential in the human body typically ranges between 2 µV and 75 µV.

  - EEG generated during mental activities (such as thinking or feeling) is even weaker, usually falling within the 2 to 10 µV range.

  - Detecting such subtle signals amidst spontaneous EEG noise can be challenging.

- Small Frequency Range:

  - The frequency range of human brain electricity (EEG) typically spans from 0.5 Hz to 40 Hz.

  - Different frequency bands (e.g., delta, theta, alpha, beta, and gamma) convey specific information about brain states and cognitive processes.

- Strong Noise:

  - EEG signals are often contaminated with various types of noise. Common sources include power frequency interference (such as 50 or 60 Hz), white noise, and spikes.

  - Additionally, EMG (electromyography) and ECG (electrocardiography) artifacts can also be present.

- Randomness and Non-Stationarity:

  - EEG signals exhibit strong randomness and non-stationarity. Due to complex factors (such as changes in brain activity, electrode placement, and external influences), EEG signals continuously vary without a clear pattern.

- Frequency Domain Characteristics:

  - The frequency domain analysis of EEG signals reveals important features. For instance, the power spectrum can highlight dominant frequency components, such as alpha, beta, theta, and delta waves.

- EEG frequency 5 bands

Surprise, Attention, engagement, combination.

- EEG preprocessing methodology

- EEG Signal Filtering and Re-sampling:

- Filtering: Involves removing unwanted noise or frequency components from EEG signals using techniques like bandpass, low-pass, or high-pass filters.

- Re-sampling: Adjusts the sampling rate of EEG data to a desired frequency.

- EEG Signal Decomposition:

- Decomposes EEG signals into their constituent components (e.g., independent components) using methods like Independent Component Analysis (ICA) or Principal Component Analysis (PCA).

- EEG Signal Processing with Graphic User Interface (GUI):

- Utilizes user-friendly interfaces to perform various EEG processing tasks, such as artifact removal, feature extraction, and visualization.

- Signal Cutting – EEG:

- Involves segmenting long EEG recordings into shorter epochs for further analysis. Useful for event-related studies.

- EEG Processing in BCI System:

- Applies EEG preprocessing techniques within Brain-Computer Interface (BCI) systems, which allow direct communication between the brain and external devices.

- Purpose, advantages, and disadvantages of EEG preprocessing methods

- Purpose

- Advantages vs. Disadvantages

- EEG signal feature extraction algorithm

- EEG signal feature classification algorithm

- Implementing EEG preprocessing methods described in the paper

- EEG Signal Filtering and Re-sampling (Python):

import brainflow

from brainflow.data_filter import DataFilter, FilterTypes, AggOperations

# Initialize the data processing module

BoardShim = brainflow.BoardShim(64, brainflow.BrainFlowInputParams())

BoardShim.enable_dev_board_logger()

# Read the data from the CSV file

data = df.values.T

# Set the sampling rate

sampling_rate = 500

# Create a DataFilter object

DataFilter.perform_lowpass(data, sampling_rate, 50.0, 2, FilterTypes.BUTTERWORTH.value)

DataFilter.perform_highpass(data, sampling_rate, 1.0, 2, FilterTypes.BUTTERWORTH.value)

- EEG Signal Processing with Graphic User Interface (GUI – MATLAB):

  - EEG Preprocessing in EEGLAB

Next Week:

- Literature Review on EEG Signal Features

- Schedule a Meeting with Wiam and Gai to Discuss EEG Signal Noise Reduction and Feature Extraction

- Continue EEG data preprocessing

- Perform Data Extraction for WMC